Biquad stability — quick reference

Form:  H(z) = (b0 + b1·z⁻¹ + b2·z⁻²) / (1 + a1·z⁻¹ + a2·z⁻²)

## 1. Hard stability conditions (must all hold)

A biquad is stable (both poles inside the unit circle) if and only if all three conditions below are true. Test each biquad after every optimisation round.

| # | Condition | Equivalent form |
| --- | --- | --- |
| 1 | ¦a2¦ < 1 | product of pole magnitudes < 1 |
| 2 | a2 > a1 − 1 | 1 − a1 + a2 > 0 (Nyquist denominator positive) |
| 3 | a2 > −a1 − 1 | 1 + a1 + a2 > 0 (DC denominator positive) |

Geometric meaning: the three conditions define a triangle in the (a1, a2) plane with corners (−2, +1), (+2, +1), and (0, −1). The stable region is the interior of this triangle.

## 2. 10-second sanity check (do this after every optimiser run)

For each biquad, compute these two numbers in your head or on paper:

- 1 + a1 + a2   (must be > 0)

- 1 − a1 + a2   (must be > 0)

- |a2|   (must be < 1)

If any of these fails: the biquad is unstable. Reject the optimiser result and re-run with tighter bounds (see §4).

## 3. Recommended VituixCAD optimiser bounds

These bounds suit normal resonant biquads (peaking EQ, shelves, low-Q crossover sections). The yellow-shaded rows are the safety-critical ones — getting these wrong is what produced the unstable solutions earlier.

| Coefficient | Min | Max | Reason |
| --- | --- | --- | --- |
| b0 | −4.0 | +4.0 | ~±12 dB gain headroom |
| b1 | −8.0 | +8.0 | can exceed b0 magnitude in mid-band |
| b2 | −4.0 | +4.0 | symmetric to b0 |
| a1 | −1.99 | +1.99 | touches stability triangle edges |
| a2 | 0.0 | +0.99 | forces complex-conjugate poles inside unit circle |

The b coefficients have no stability impact (they place zeros, not poles). The values above are sensible for ±12 dB EQ work; widen them if you need higher gains.

## 4. When the optimiser still finds an unstable solution

Rectangular per-coefficient bounds cannot fully describe the triangular stable region, so the optimiser can still slip out at the corners. If a biquad fails the §2 check:

- Tighten a1 bounds incrementally on that biquad: ±1.95, then ±1.90, then ±1.80, until a stable solution is found.

- If the optimiser cannot fit the target with a stable biquad, that slot wants something a single resonant biquad cannot produce — split it into two cascaded 1st-order sections (each with a2 = 0, one real pole).

- If the original biquad has both poles on the real axis (no imaginary part), it is already two 1st-order sections fused together. Replace it with a low-shelf/high-shelf parameter block, or with two adjacent 1st-order biquad slots.

## 5. Empirical guideline from the conversion work

- a2 between +0.84 and +0.98 → healthy resonant biquads. These typically need only minor optimiser tuning after sample-rate conversion.

- a2 ≤ +0.30 → caution. These tend to be the unstable ones in practice, and often indicate that the original filter wasn’t really a resonant biquad (probably two 1st-order sections in disguise).

## 6. Other quick checks worth doing

- b0 sign: if b0 is negative, the high-frequency portion of that biquad inverts polarity. Not automatically wrong, but check whether you actually wanted that.

- Pole frequency: for complex poles, f ≈ (fs / 2π) · arccos(−a1 / (2·√a2)).  At 96 kHz, a complex pole pair with a1 = −1.97, a2 = +0.98 corresponds to roughly 1.5 kHz.

- Cascade order: in fixed-point implementations (e.g. ADAU1701 in 5.23 format), the order of biquads in the chain affects internal headroom. If one biquad has a high peak gain, put it later in the chain rather than first, so it does not amplify quantisation noise from earlier stages.

Reference form used throughout: VituixCAD “Generic” export, SigmaStudio General 2nd-order cell. No sign flipping on a1, a2 between the two tools.