	M3N Full-Frequency

	Nominal Impedance (Z)(Ω) : 
	8 

	Resonance Frequency (Fs)(Hz) : 
	91 

	Nominal Power Handling (Pnom)(W) : 
	15 

	Max Power Handling (Pmax)(W) : 
	30 

	Sensitivity (2.83v/1m)(dB) : 
	82 

	Weight (M)(Kg) : 
	0.55 

	VC Diameter (mm) : 
	20 

	DC (Re)(Ω) : 
	6.5 

	VC Length (H)(mm) : 
	9.0 

	VC Former : 
	CCAW 

	VC Frame : 
	Kapton 

	Magnet System : 
	Shielded 

	Magnet Former : 
	Ferrite 

	Force Factor (BL)(N/A) : 
	3.0 

	Gap Height (He)(mm) : 
	3.0 

	Linear Excursion (Xmax)(mm) : 
	3.0 

	Suspension Compliance (Cms)(uM/N) : 
	1172 

	Mechanical Q (Qms) : 
	3.57 

	Electrical Q (Qes) : 
	1.03 

	Total Q (Qts) : 
	0.80 

	Moving Mass (Mms)(g) : 
	2.6 

	Effective Piston Area (Sd)(m2) : 
	0.0030 

	Equivalent Air Volume (Vas)(L) : 
	1.5 

	Cabinet Type : 
	Sealed 

	Recommended Box Volume(Vb)(L) : 
	1.6 

	-3dB Cut-Off Frequency(F3)(Hz) : 
	97 


