	FRS 8 M 8 OHM

	

	Art. No. 2001

8 cm (3.3") HiFi fullrange speaker with high force factor and good sensitivity. Special developed for use in column-speakers and small bass-reflex cabinets, e.g. in miniature subwoofer / satellite systems.

Typical applications :
- Column speakers
- Mini hi-fi systems

Attributes :
- Moisturised cone
- Metal basket


Used in / Accessories:
GRILLE FRS 8
GRILLE 8 ES
	


	



sketch
 





frequency- and impedance response
 





 

 


	Technical Data:

Rated power

30 W

Maximum power

50 W

Nominal impedance Z

8 Ohm

Frequency response

100–20000 Hz

Mean sound pressure level

88 dB (1 W/1 m)

Opening angle (-6 dB)

147°/4000 Hz

Excursion limit

+/−2,5 mm

Resonance frequency fs

125 Hz

Magnetic induction

1,2 T

Magnetic flux

225 µWb

Height of front pole-plate

4 mm

Voice coil diameter

20 mm

Height of winding

4,5 mm

Cutout diameter

75 mm

Net weight

0,5 kg

D.C. resistance Rdc

7,2 Ohm

Mechanical Q factor Qms

3,24

Electrical Q factor Qes

0,58

Total Q factor Qts

0,49

Equivalent volume Vas

1,1 l

Effective piston area Sd

29 cm²

Dynamically moved mass Mms

1,8 g

Force factor Bxl

4,0 T m

Inductance of the voice coil L

0,3 mH

Temperature range

−25 ... 70 °C

Similar Products

F 8 SC 8 OHM 

FR 8 4 OHM 

FR 87 4 OHM 

FR 8 8 OHM 

FR 8 JS 8 OHM 

FR 8 WP 4 OHM (black) 

FR 8 WP 4 OHM (white) 

FR 8 WP 8 OHM (black) 

FR 8 WP 8 OHM (white) 

FRS 8 4 OHM 

FRS 8 8 OHM 

SC 8 N 8 OHM 

SL 87 WPM 4 OHM 

SL 87 WPM 8 OHM 

SL 87 XA 8 OHM 

 



	

directivity pattern
 




